[image: image1.jpg]DEPARTMENT OF LABOR, LICENSING
AND REGULATION




OCCUPATIONAL HEALTH STANDARD

Lead In Construction

The Federal Lead Exposure in Construction Standard, 29 CFR 1926.62 with Maryland Amendments, requires that before beginning any construction work which may result in lead exposure, an employer must determine if any employee may be exposed to lead at or above the action level (30µg/m³).  This determination must be in writing and be posted.  If any employee may be exposed at or above the action level, the employer must conduct air sampling at the start of the operation that may involve lead exposure.  The major requirements of the Lead in Construction Standard are detailed below. 
	Any Airborne Lead
	At or Above Action Level (30µg/m³)
	Above PEL (50µg/m³)
	 

	 
	 
	 
	CONDUCT EXPOSURE MONITORING

	 
	X
	X
	 - at start of job

	 
	X
	 
	 - every six (6) months

	 
	 
	X
	 - every quarter

	 
	X
	X
	 - when job change may result in new or additional exposure

	X
	X
	X
	 - if employee complains of symptoms related to lead exposure

	 
	 
	 
	Whenever exposure monitoring is performed, employee must be provided with written notice of results

	 
	 
	 
	ENGINEERING AND WORK PRACTICE CONTROLS

	 
	 
	X
	 - use when feasible to reduce exposure below PEL

	 
	 
	 
	COMPLINACE PROGRAM

	 
	 
	X
	 - develop written compliance program and review every six (6) months)

	 
	 
	 
	PROVIDE RESPIRATORY PROTECTION

	 
	 
	X
	 - when exposure exceeds PEL

	 
	 
	X
	 - as interim measure

	 
	 
	X
	 - to supplement engineering and work practice controls

	 
	 
	X
	 - when controls are not feasible

	 
	 
	X
	 - upon employee request

	 
	 
	 
	When respirators are provided, a respiratory protection program in accordance with 29CFR 1910.134 must be established and fit testing must be conducted.  A medical examination must be provided if an employee exhibits difficulty breathing during respirator fit test or use.  An employer must provide a powered air-purifying respirator at the employee's request.

	 
	 
	 
	PROTECTIVE CLOTHING AND EQUIPMENT

	 
	 
	X
	 - provide appropriate protective equipment and clothing

	 
	 
	X
	 - clean clothing weekly (daily if exposure above 200µg/m³)

	 
	 
	X
	 - assure protective clothing removed at end of shift

	 
	 
	X
	 - assure appropriate laundering or disposal

	 
	 
	X
	 - clean and repair equipment

	 
	 
	 
	Protective clothing and equipment must also be provided when the possibility of skin or eye irritation exists.


	Any Airborne Lead
	At or Above Action Level (30µg/m³)
	Above PEL (50µg/m³)
	 

	 
	 
	 
	MAINTENANCE OF SURFACES

	X
	X
	X
	 - maintain all surfaces as free of lead as possible

	X
	X
	X
	 - prohibit cleaning by compressed air

	X
	X
	X
	 - use vacuuming or other equally effective cleaning methods

	X
	X
	X
	 - use wet methods when vacuuming not feasible

	 
	 
	 
	EATING, DRINKING AND SMOKING

	 
	 
	X
	 - prohibit eating, drinking and smoking in job area

	 
	 
	X
	 - provide eating and drinking area

	 
	 
	X
	 - assure employees wash prior to eating or drinking

	 
	 
	X
	 - assure employees do not enter eating area in lead contaminated clothing

	 
	 
	 
	CHANGE AREAS AND STORAGE

	 
	 
	X
	 - provide changing areas and storage

	 
	 
	X
	 - assure employees do not leave job area in contaminated clothes

	 
	 
	 
	SHOWER FACILITIES

	 
	 
	X
	 - provide shower facilities where feasible

	 
	 
	X
	 - when provided, assure employees use

	 
	 
	X
	 - provide adequate supply of cleansing agents and towels

	 
	 
	 
	HAND WASHING FACILITIES

	X
	X
	X
	 - provide adequate hand washing facilities (29CFR 1926.51(f))

	 
	 
	X
	 - assure employees wash at end of each shift when showers not available

	 
	 
	 
	LAVATORY FACILITIES

	X
	X
	X
	 - provide lavatory facilities according to 29CFR 1926.51©

	 
	 
	 
	MEDICAL SURVEILLANCE

	 
	X
	X
	 - institute medical surveillance program

	 
	 
	 
	 Biological monitoring (Blood Lead + ZPP or FEP Levels

	 
	X
	X
	 - prior to assignment

	 
	X
	X
	 - every two (2) months for first six (6) of exposure

	 
	X
	X
	 - written notification of results to employee

	 
	 
	 
	Medical examination must be provided when exposure is above action level and employee has developed signs or symptoms associated with lead intoxication, desires advice on effects of exposure on ability to procreate, or employee's blood lead level is at or above 40µg/100g. A medical examination must also be provided when an employee exhibits difficulty breathing during respirator fit test or use. 

	 
	 
	 
	PROVIDE MEDICAL REMOVAL AND PROTECTION

	 
	X
	X
	 - if blood level is at or above 50µg/100g

	 
	X
	X
	 - of indicated by a final medical determination

	 
	 
	 
	INFORMING EMPLOYEES

	X
	X
	X
	 - inform employees of standard

	X
	X
	X
	 - make available a copy of standard

	 
	 
	 
	TRAINING

	 
	X
	X
	 - provide training program

	 
	 
	 
	Training must also be made available if the possibility of skin or eye irritation exists

	 
	 
	 
	SIGNAGE

	 
	 
	X
	 - post warning signs

	 
	 
	 
	MAINTAIN RECORDS OF:

	X
	X
	X
	 - initial determination

	X
	X
	X
	 - exposure monitoring

	 
	X
	X
	 - medical surveillance

	 
	X
	X
	 - medical removal


This chart is intended to summarize the Federal Lead Exposure in Construction Standard, 29 CFR 1926.62 with Maryland Amendments, and is not to be interpreted as the complete requirements under the standard.  For a complete copy of the standard contact MOSH Training & Education at 410-880-4970 or 301-483-8406.
02/09

